CN ASSIGNMENT 6

Q1. To configure and verify routing using RIP Version 2 protocol in a network.

Commands for Router 1

enable

conf t

interface gig0/0/0

ip address 100.1.1.2 255.255.255.0

clock rate 64000

no shutdown

interface gig0/0/1

ip address 20.1.1.1 255.255.255.0

no shutdown

router rip version 2

no auto-summary

network 100.1.1.0

network 20.1.1.0

end

Commands for Router 2

enable

conf t

interface gig0/0/0

ip address 100.1.1.1 255.255.255.0

no shutdown

interface gig0/0/1

ip address 172.16.2.1 255.255.0.0

no shutdown

router rip version 2

no auto-summary

network 100.1.1.0

network 172.16.0.0

end

Commands for Router 3

enable

conf t

interface gig0/0/0

ip address 20.1.1.2 255.255.255.0

no shutdown

interface gig0/0/1

ip address 10.2.2.1 255.255.255.0

no shutdown

router rip version 2

no auto-summary

network 20.1.1.0

network 10.2.2.0

end