CN ASSIGNMENT 4

Q.1 Create a pool on the DHCP Server router, named “xyz” and follow all the requirements.

1. Client should use the DNS server with the IP address: 1.2.3.4 2. Client should use the network 192.168.10.0/24

3. Client should renew their IP address after every 3 hours

4. Make sure, Router does not respond to any BOOTP

5. Configure the Client so that it will obtain IP from DHCP server.

Commands for router:

enable

configure terminal

interface gig 0/0/0

ip address 192.168.10.1 255.255.255.0

no shut

exit

Exclude IP for router itself

ip dhcp excluded-address 192.168.10.1

Create DHCP Pool XYZ

ip dhcp pool xyz

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 1.2.3.4

lease 0 3   ! (0 days, 3 hours)

exit

Disable BOOTP Services

no ip bootp server

Save Configuration

end

write memory