# CN ASSIGNMENT 5

Q1. Configure Static Route in Cisco packet tracer

Commands for Router 1:

R1> enable

R1# configure terminal

R1(config)# interface gig 0/0/0

R1(config-if)# ip address 192.168.1.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface gig 0/0/1

R1(config-if)# ip address 10.0.0.1 255.255.255.252

R1(config-if)# clock rate 64000

R1(config-if)# no shutdown

R1(config-if)# exit

Commands for Router 2:

R2> enable

R2# configure terminal

R2(config)# interface gig 0/0/0

R2(config-if)# ip address 10.0.0.2 255.255.255.252

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# interface gig 0/0/1

R2(config-if)# ip address 172.16.0.1 255.255.0.0

R2(config-if)# no shutdown

R2(config-if)# exit

Verify interface configuration

show ip interface brief

Display routing table before static routing

show ip route

Configure static route on Router R1

R1(config)# ip route 172.16.0.0 255.255.0.0 10.0.0.2

Configure static route on Router R2

R2(config)# ip route 192.168.1.0 255.255.255.0 10.0.0.1

Q2. Suppose you want to create 3 subnets of a network having network address 198.168.1.0. In another network with network address 220.168.2.0, you want to create 2 subnets. For this scenario configure the end devices and routers accordingly. Connect two different networks (both are subnetted) through static routing and show the message transmission among hosts of the two networks.

Commands for Router 1

enable

configure terminal

interface gig0/0/0

ip address 198.168.1.1 255.255.255.192

no shutdown

interface gig0/0/1

ip address 198.168.1.65 255.255.255.192

no shutdown

interface gig0/0/2

ip address 198.168.1.129 255.255.255.192

no shutdown

interface serial0/0/0

ip address 10.0.0.1 255.255.255.252

clock rate 64000

no shutdown

exit

ip route 220.168.2.0 255.255.255.128 10.0.0.2

ip route 220.168.2.128 255.255.255.128 10.0.0.2

end

Commands for Router 2

enable

configure terminal

interface gig0/0/0

ip address 220.168.2.1 255.255.255.128

no shutdown

interface gig0/0/1

ip address 220.168.2.129 255.255.255.128

no shutdown

interface serial0/0/0

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

ip route 198.168.1.0 255.255.255.192 10.0.0.1

ip route 198.168.1.64 255.255.255.192 10.0.0.1

ip route 198.168.1.128 255.255.255.192 10.0.0.1

end

Q3. Connect two networks having network address 192.168.3.0 and 224.152.1.0 respectively. Network 1 having 4 and network two having 3 hosts. Connect two different networks through a router. For this scenario configure the end devices and routers accordingly and show the message transmission among hosts of the two networks.

Command for Router 1

enable

configure terminal

interface gig0/0/0

ip address 192.168.3.1 255.255.255.248

no shutdown

interface gig0/0/1

ip address 224.152.1.1 255.255.255.248

no shutdown

end