# CN ASSIGNMENT 1

# To capture live network packets using Wireshark and identify basic network protocols such as TCP, UDP, IP, and DNS.

Step 1: Launch Wireshark

  - Open Wireshark

  - You will see a list of network interfaces

    - Wi-Fi

    - Ethernet

# Step 2: Start Packet Capture

  - Double-click on Wi-Fi (or active interface)

  - Packet capture will start immediately

# Step 3: Generate Network Traffic

Perform any of the following:

  - Open a web browser

  - Visit any website (e.g., google.com)

  - Open Command Prompt and run:

  - ping google.com

# Step 4: Stop Capture

  - Click the red Stop button

  - Captured packets will remain on the screen

# Step 5: Apply Basic Filters

Use the Display Filter bar:

# Filter	Purpose

ip	Show IP packets

tcp	Show TCP packets

udp	Show UDP packets

dns	Show DNS packets

icmp	Show ping packets

# Step 6: Analyze a Packet

- Click on any packet

- Observe:

  - Ethernet header

  - IP header

  - TCP/UDP header

  - Payload data